H8-AM9511 Arithmetic Processor Card
Jumper Definitions & Assembly Notes

JP1, JP2 – Clock select
Install only one jumper in these two positions

On-board oscillator	JP1 “Int Osc”	(no clock division)
Clock from H8 bus pin 22
	2MHz bus	JP1 “Ext”	(no clock division)
	4MHz bus	JP2 “4”		(divide bus clock by 2)
	8MHz bus	JP2 “8”		(divide bus clock by 4)

JP3 – Interrupt Source Select (see AM9511 datasheet, not needed for normal (polled) operation)
	END		Pins 1-2 (left)
	SVREQ		Pins 2-3 (right)

JP4 – IORQ Source Select
	Z80 IORQ	“/IORQ”	Used for v3+ Z80 cards that put Z80 IORQ on bus pin 24

8080		“FAKE”		Used for 8080 or older Z80 cards (Heath, Trionyx)

IORQ is synthesized from 8080 IOW + IOR

P7 – I/O Select
	Install one jumper at 200Q, 64Q, or 62Q to select board address

Install one jumper at “Soft Reset” to use software reset feature (optional)

A write to any of ports 66Q,67Q,202Q or 203Q will reset the AM9511 APU

P8 – INT Select
	Do not install a jumper here if using polled operation (default)

You may choose to use INT 3 (bottom position) through INT 6 (top position)

Assembly Notes

Lead spacing for all ceramic capacitors is 0.1”

AMD AM9511 and Intel C8231A APUs are interchangeable

All APUs will operate with 2MHz clock, 3- and 4-MHz versions were also produced (see datasheet)

APUs run very hot – an adhesive-attached heatsink is a good idea (see BOM for one suggestion)