How to program 82C55 in mode 2 to talk to the ESP32 module

By: Norberto Collado Rev 1.0 10/17/2025

Registers:

- 0xD2 = Port C address (byte level)

- 0xD0 = Address to read/write from the EP32 module

- 0xD3 = Address of the 82C55 control word.

- 0xD3 = address to set/reset PORT C bits individually (bit level)

Writing to the ESP32 module

- Power-on H89 system with H89-FujiNet board installed

- Load onto ESP32 “fujinet-H89-v1.5.1-five-nops.zip” using FujiNet Flasher

  - https://github.com/FujiNetWIFI/fujinet-flasher/releases/

Note: Put the H89 ESP32 in “Download Boot” to flash the FW with FujiNet Loader.

- Out 0xD2, 0x04 	/* EN/RESET = 1, IO0 = 0, IO2 = 0 */

- Out 0xD2, 0x00 	/* strobe the reset line */

- Out 0xD2, 0x04 	/* EN/RESET = 1, IO0 = 0, IO2 = 0 */

When done, put it onto SPI boot mode:

- Out 0xD2, 0x06 	/* EN/RESET = 1, IO0 = 1, IO2 = 0 */

- Out 0xD2, 0x02 	/* strobe the reset line */

- Out 0xD2, 0x06 	/* EN/RESET = 1, IO0 = 1, IO2 = 0 */

- Setup 82C55 to Mode 2:

  - out 0xD3, 0xC0 - /* mode 2, port B mode 1 out, port C0-2 out */

- Setup LVC245 direction to send data to the ESP32

  - Out 0xD3, 0x01 /* SET “DIR= 1” */

  - Out 0xD3, 0x02 /* Set “/OE=0” */

- Setup “EN” to be High (1) to enable ESP32

  - Out 0xD3, 0x05 /* EN = HIGH */

- Send a data byte to the ESP32

  - Out 0xD0, 0xAA

  - ESP32 receives the data.

- Go to step 8 to send another byte.

Reading from the ESP32 module

- Load onto ESP32 “fujinet-H89-v1.5.1-tx-invert-the-bits.zip” using FujiNet Flasher

  - https://github.com/FujiNetWIFI/fujinet-flasher/releases/

Note: Put the H89 ESP32 in “Download Boot” to flash the FW with FujiNet Loader.

- Out 0xD2, 0x04 	/* EN/RESET = 1, IO0 = 0, IO2 = 0 */

- Out 0xD2, 0x00 	/* strobe the reset line */

- Out 0xD2, 0x04 	/* EN/RESET = 1, IO0 = 0, IO2 = 0 */

When done, put it onto SPI boot mode:

- Out 0xD2, 0x06 	/* EN/RESET = 1, IO0 = 1, IO2 = 0 */

- Out 0xD2, 0x02 	/* strobe the reset line */

- Out 0xD2, 0x06 	/* EN/RESET = 1, IO0 = 1, IO2 = 0 */

- Setup LVC for Input from the ESP32.

  - Out 0xD3, 0x00 /* Set DIR =0 */

  - Out 0xD3, 0x02 /* Set “/OE=0” */

- Reset ESP32

- Read a byte from the ESP32

  - In 0xD0

    - H89 receives 0xAA and so on…